2015 UIC Project Form

API number
087-01578

UIC permit
2D0871578


Current operator
GENESIS HYDROCARBON

Well in operation since
1967

Location of well
Roane county

Formation(s)
Salt sands, Big Injun

Major Issues

1) Permit expired June 2004, injection was reported on

WR40s through 2007.

2) January 2014 NOV is for expired permit, operation without

a permit is prohibited. That NOV was abated and a new NOV

was issued in February for the well being abandoned with an additional

order to cease operations.
MIT

Most recent
04/21/1998 
other passed (checked fluid level)

Previous
02/03/1993 


Permitting

Application



Expiration of permit
11/13/2013


Previous application
9/2/2008


Expiration
6/9/2004


Inspections

02/11/2014 
NOV 22-6-19
11/9/2011
no cement on surface casing,


permit needs to be cancelled

5/6/2009


9/4/2007


4/6/2001


1/26/2000


6/22/1999


NOVS

02/11/2014 
NOV 22-6-19

01/22/2014
NOV 47CSR13.13.1a  -- injection without a permit

WR-40s

No injection from 2011 to present.

Comments

Gravity fed injection well (injection through tubing annulus) that is also producing through tubing. It appears that injection is also taking place into the Big Injun which is the production zone. No oil production reported after 2008.

Completion report seems to show no cement behind surface and intermediate casing, only cement behind production casing. 

Abandoned.





NOV for being abandoned, without evidence of plugging permit application resulting from NOV.





Completion report does not show cement behind surface casing.








